(B&G)

Compressed Sprayer Maintenance

Standard Operating Procedure (SOP): Basic Annual

Purpose

This Standard Operating Procedure (SOP) outlines the procedure for basic annual maintenance of a 1-gallon compressed air sprayer. The instructions herein apply to the B&G 1-gallon sprayer; other sprayer types may be out of the purview of this document. See your manufacturer’s recommendations for further information.

Related Information:

- Notify your direct manager immediately if your sprayer is broken or damaged. Repairs requiring replacement parts are outside the purview of this SOP.

- This SOP covers routine maintenance that may need to be performed due to daily wear from normal use. Sprayers may become damaged beyond the purview of this SOP from heavy use or rough handling. Keep your 1-gallon sprayer secure in a location it will not be damaged while driving and between treatments.

- Repairs beyond the scope of this SOP should be performed to the manufacturer’s recommendations. Consult the manufacturer’s manual or information for any complicated repair.

- These procedures should be performed annually at a minimum; the frequency can be increased to biannually or as needed for frequently used equipment.

Procedure:

- Annual or Biannual Service or as needed:

    - Inspection/Replacement of worn or damaged parts:

- Check valves, seals, gaskets, and the plunger cup for tears and wear.

- Lubricate plunger cup with petroleum-type grease (NOT an oil spray)

- Check the hose for wear or damage, especially around the threaded connector

- Replace parts if needed

    - Check nozzle for calibration:

  - Place a known quantity of liquid in the tank

  - Pressurize the tank to the pressure recommended by the manufacturer of your nozzle or sprayer

  - Spray the liquid, and compare the actual output to the expected output provided by the manufacturer.

  - If more than the acceptable difference from the manufacturer's standard, replace the nozzle. If the manufacturer does not provide an acceptable difference, replace if there is more than 10% deviation from the standard.

  - Nozzles may have to be replaced approximately every two years, with the exact replacement schedule determined by the frequency of use.

Conclusion

By following this SOP, specialists can perform basic annual maintenance on their 1-gallon B&G compressed air sprayer.